EXPERIMENT I

INTRODUCTION TO LABORATORY EQUIPMENT FOR CHARACTERIZATION OFBASIC ELECTRICAL PARAMETERS

Fall - 2023/2024

EEE 281 – Electrical Circuits Lab 1 Report

Submitted By:	Eda İslam

Yağmur Can

2585081

2637536

2585081

2637536

Student ID:

Date:   26/10/2023

Sections of the Report

The laboratory report has three primary sections:

- Objective

- Results

- Conclusions

### Objective:

In this experiment,

- i. We learned how to read color codes of resistors and determine the value of the resistor.

  - We learned how to use a potentiometer.

  - We learned how we can use multimeter for resistance measurements.

- We learned how to work with a power source and multimeter for parametric I-V analysis.

- We learned the simple usage of oscilloscope.

EQUIPMENT LIST:

- Function Generator

- DC Power Supply

- Digital Multimeter

- Digital Oscilloscope

- Carbon Resistors (1 kΩ, 4.7 kΩ, 16 kΩ)

- Potentiometers (5 kΩ POT and 500 kΩ POT)

### Results:

## 1a)  Comparison of Resistance Measurements with Specifications

| Resistor | Spec value (kΩ) | Spec tolerance | Measured | Error (%) with |
| --- | --- | --- | --- | --- |
| # |  | (%) | value (kΩ) | respect to spec |
| 1 | 18 | ±5 | 17.90 | 0.55 |
| 2 | 4.7 | ±5 | 4.71 | 0.21 |
| 3 | 1 | ±5 | 0.98 | 2 |

- We used color code in order to find Spec value (kΩ).

- We used digital multimeter to find measured value (kΩ).

- We use the following equation to find the error rates:

# % error = ((spec value - measured value) / spec value) * 100)

## 1b) Comparison of Resistance Measurements with Specifications (With POTS)

| POT # | Spec | Measured | Error |
| --- | --- | --- | --- |
|  | Resistance | Max value | (%) with |
|  | Range (kΩ) | (kΩ) | respect |
|  |  |  | to spec |
| 1 | 5 | 4.6 | 8 |
| 2 | 500 | 489 | 2.2 |

- In order to record the spec resistance range, we read the value that is written onto potentiometers.

- After arranging potentiometer to its maximum position, we measured the max value by using digital multimeter.

- In order to calculate error with respect to spec we used the equation following:

# % error = ((spec value - measured value) / spec value) * 100)

- POT # 1 has an error rate 8 which is quite high but it is on the acceptable range which is ±10%.

### 2-) Comparison between Resistance Measurement  Methods based on Measurement Accuracy

We set up the circuit on the right on the breadboard in order to calculate resistor of 4.7kΩ resistor using the DC power supply. We arranged the voltage of the power supply to 2,4,6,8 and recorded the current for each voltage

|  | Voltage (V) | Measured Current (A) | Spec value (kΩ) | Spec tolerance | Measured value (kΩ) using multimeter (part b above) | Measured value (kΩ) using I-V method (part a above) | Error (%) of the resistance measurement using I-V method with respect to spec |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | 0.00011 | 18 | ±5% | 17.90 | 18.2 | 1.11 |
|  | 4 | 0.00022 |  |  |  |  |  |
|  | 6 | 0.00033 |  |  |  |  |  |
|  | 8 | 0.00044 |  |  |  |  |  |

  - In order to record the wanted values in part a and b of the second part we measured current with multimeter with different voltages (2,4,6,8).

  - First, we plot the graph of I-V then we used the slope of the graph to find the resistance of the resistor. By using ohm’s law, we calculated the resistance. By using the formula R=V/I, which indicates the slope of the graph (slope = (y2 - y1) / (x2 - x1)) we choose two random points then we calculated the resistance. Then we have an equation: resistance = V / I = slope = (8 – 2) / (0.00044 – 0.00011) = 18.2 kΩ

  - The resistance value that we measured with the multimeter is 17.9, by using the slope equation we found the resistance value 18.2, which is more accurate than the value that we measured using multimeter. Differences between two values stems from the measurement errors, cable’s resistance etc.  The resistance value that we find using slope of the V-I graph is more accurate than the multimeter value because we used multiple data values.

### Conclusion:

- We studied how we can use DC Power Supply and Digital Multimeter.

- We studied how to find resistance of a resistor using color code.

- We studied measuring resistance of resistors and POTS with multimeter.

- We studied finding the resistance value by using voltage and current values from the graph (slope).

- As a conclusion, we realized that measuring the resistance by using multiple data is more certain than using only the multimeter.